## Configuration of each node

This table shows configuration details of each node in the array.  The following section shows all specially configured nodes and groups of nodes.

| Computers / Nodes | Comm Ports Present | I/O Addressing & Type |  | Reset | ROM Type | Special Config |
| --- | --- | --- | --- | --- | --- | --- |
| 001 | RDL- | U | SERDES | COLD | SERDES Boot |  |
| 100 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 200 | RD-U | L | GPIOx1 | rd-u | 1-wire |  |
| 300 | RD-U | L | GPIOx2 | COLD | Synchronous Boot |  |
| 500 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 600 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 701 | RDL- | U | SERDES | COLD | SERDES Boot |  |
| 705 | RDL- | U | GPIOx4 | COLD | SPI Boot |  |
| 708 | RDL- | U | GPIOx2 | COLD | Asynchronous Boot |  |
| 709 | RDL- | U | Analog | rdl- | Analog | Shared pin |
| 713 | RDL- | U | Analog | rdl- | Analog |  |
| 715 | RDL- | U | GPIOx1 | rdl- | Basic |  |
| 717 | RD-- | U | Analog | rd-- | Analog |  |
| 617 | RD-U | L | Analog | rd-u | Analog | Shared pin |
| 517 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 417 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 317 | RD-U | L | GPIOx1 | rd-u | Basic |  |
| 217 | RD-U | L | GPIOx1 | rd-u | Basic | Shared pin |
| 117 | RD-U | L | Analog | rd-u | Analog |  |
| 009 | RDL- | U | Parallel Bus | rdl- | SDRAM Addr | Phantom pin group, special ROM |
| 008 | RDL- | U | GPIOx4 | rdl- | SDRAM Ctl |  |
| 007 | RDL- | U | Parallel Bus | rdl- | SDRAM Data |  |
| 105 | RDLU |  |  | rdlu | eForth Bitsy | Special ROM |
| 106 | RDLU |  |  | rdlu | eForth stack |  |
| 107 | RDLU |  |  | rdlu | SDRAM mux |  |
| 108 | RDLU |  |  | rdlu | SDRAM idle |  |
| 000, 017, 700 | RD-- |  |  | rd-- | Basic |  |
| 002, 003, 004, 005, 006, 010, 011, 012, 013, 014, 015, 016, 702, 703, 704, 706, 707, 710, 711, 712, 714, 716 | RDL- |  |  | rdl- | Basic |  |
| 400 | RD-U |  |  | rd-u | Basic |  |
| 101-04, 109-16, 201-16, 301-16, 401-16, 501-16, 601-16 | RDLU |  |  | rdlu | Basic |  |