## GA144 Signal map

These tables identify header pins or holes at which each chip's signals may be found.

### Host Chip

| Type | Name | Pin | Access | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SRAM Data Bus | d00 | 1 | D00 | Bits 0 through 17 of node 007 UP port. General purpose bidirectional parallel bus. Bits 0 through 15 are connected to the SRAM BGA and probe access is not supported except for bit 0 to probe timing. Mapping to SRAM data lines is randomized for best layout. Bits 16 and 17 are pulled down so that they will read as zero. |  |  |  |
|  | d01 | 2 | None |  |  |  |  |
|  | d02 | 3 |  |  |  |  |  |
|  | d03 | 8 |  |  |  |  |  |
|  | d04 | 9 |  |  |  |  |  |
|  | d05 | 10 |  |  |  |  |  |
|  | d06 | 11 |  |  |  |  |  |
|  | d07 | 12 |  |  |  |  |  |
|  | d08 | 13 |  |  |  |  |  |
|  | d09 | 16 |  |  |  |  |  |
|  | d10 | 21 |  |  |  |  |  |
|  | d11 | 22 |  |  |  |  |  |
|  | d12 | 23 |  |  |  |  |  |
|  | d13 | 24 |  |  |  |  |  |
|  | d14 | 25 |  |  |  |  |  |
|  | d15 | 30 |  |  |  |  |  |
|  | d16 | 31 | R14 |  |  |  |  |
|  | d17 | 32 | R13 |  |  |  |  |
| GPIO | 008.17 | 33 | None | General purpose 4-pin node used for SRAM control (1,3) and high order address lines (5,17). Pins 1 and 3 are pulled up so that the SRAM is made inactive when chip is reset. |  |  |  |
|  | 008.5 | 34 |  |  |  |  |  |
|  | 008.3 | 35 | WE- |  |  |  |  |
|  | 008.1 | 36 | CE- |  |  |  |  |
| SRAM Address Bus | a17 | 37 | None | Bits 17 through 0 of node 009 UP port. General purpose bidirectional parallel bus. Bits 0 through 17 are connected to the SRAM BGA and probe access is not supported except for bit 0 to probe timing. Mapping to the low order 17 SRAM address lines is randomized for best layout. A17 must be mapped directly so that the layout will accommodate chips with 128k or fewer words. |  |  |  |
|  | a16 | 38 |  |  |  |  |  |
|  | a15 | 39 |  |  |  |  |  |
|  | a14 | 42 |  |  |  |  |  |
|  | a13 | 43 |  |  |  |  |  |
|  | a12 | 44 |  |  |  |  |  |
|  | a11 | 45 |  |  |  |  |  |
|  | a10 | 46 |  |  |  |  |  |
|  | a09 | 53 |  |  |  |  |  |
|  | a08 | 54 |  |  |  |  |  |
|  | a07 | 55 |  |  |  |  |  |
|  | a06 | 56 |  |  |  |  |  |
|  | a05 | 57 |  |  |  |  |  |
|  | a04 | 58 |  |  |  |  |  |
|  | a03 | 65 |  |  |  |  |  |
|  | a02 | 66 |  |  |  |  |  |
|  | a01 | 67 |  |  |  |  |  |
|  | a00 | 68 | A00 |  |  |  |  |
| SERDES | 001.17 | 27 | J75 | Node 001 Clock |  |  | Available at dedicated SMA connector hole patterns.. |
|  | 001.1 | 26 | J74 | Node 001 Data |  |  |  |
| SERDES | 701.17 | 86 | S17 | Node 701 Clock |  |  | Connected to Target node 001 SERDES. Both chips reset to SDERDES boot. |
|  | 701.1 | 87 | S1 | Node 701 Data |  |  |  |
| GPIO | 300.17 | 14 | J35.1 | Sync clock | General purpose 2-pin node. ROM supports synchronous boot. May be connected to Target node 300. |  |  |
|  | 300.1 | 15 | J34.1 | Sync data |  |  |  |
| GPIO | 708.17 | 78 | J23.2 | Rx Input |  | General purpose 2-pin node. ROM supports asynchronous boot. May be connected to USB port A for IDE operations. |  |
|  | 708.1 | 79 | J23.4 | Tx Out |  |  |  |
| GPIO | 705.17 | 85 | DI | Data In |  | General purpose 4-pin node. Normally used for boot and/or read/write on SPI Flash and/or mass storage such as MMC depending on jumpers. May also be free for application use. When MMC selected, SS- and other signals are on J40. |  |
|  | 705.5 | 84 | DO | Data Out |  |  |  |
|  | 705.3 | 81 | SS- | Chip Enable- |  |  |  |
|  | 705.1 | 80 | SCK | Clock |  |  |  |
| GPIO | 100.17 | 20 | J23.8 | 1-pin GPIO nodes. May be connected to USB port B for use with high level Virtual Machines. |  |  |  |
|  | 200.17 | 18 | J23.6 |  |  |  |  |
|  | 500.17 | 7 | J22.1 | 1-pin GPIO node. May be used to reset the Target chip. |  |  |  |
|  | 600.17 | 6 | J39.1 | 1-pin GPIO node. May be used in selecting expanded SPI bus. |  |  |  |
|  | 317.17 | 52 | J21.5 | 1-pin GPIO nodes. Available for application use. |  |  |  |
|  | 417.17 | 59 | J21.4 |  |  |  |  |
| Analog In | 709.ai | 76 | J27.3 | Analog nodes whose I/O is powered by separate VDDA bus. Available for application use. |  |  |  |
| Analog Out | 709.ao | 77 | J27.2 |  |  |  |  |
| Analog In | 713.ai | 73 | J27.4 |  |  |  |  |
| Analog Out | 713.ao | 72 | J27.5 |  |  |  |  |
| Analog In | 717.ai | 69 | J27.8 |  |  |  |  |
| Analog Out | 717.ao | 70 | J27.7 |  |  |  |  |
| GPIO | 715.17 | 71 | J27.6 | General purpose 1-pin node whose pin is shared (read only) by the above analog nodes and may be used by them for timing or other purposes. |  |  |  |
| Analog In | 617.ai | 61 | J21.2 | Analog node whose I/O is powered by VDDI bus. |  |  |  |
| Analog Out | 617.ao | 63 | J21.1 |  |  |  |  |
| GPIO | 517.17 | 60 | J21.3 | General purpose 1-pin node whose pin is shared (read only) by Analog 617. |  |  |  |
| Analog In | 117.ai | 48 | J21.8 | Analog node whose I/O is powered by VDDI bus. |  |  |  |
| Analog Out | 117.ao | 50 | J21.7 |  |  |  |  |
| GPIO | 217.17 | 51 | J21.6 | General purpose 1-pin node whose pin is shared (read only) by Analog 117. |  |  |  |
| Input | RESET- | 88 | J20.1 | Reset signal, active low. Also pin J20.2. |  |  |  |
| Power | VDDC | 5 | J10.2 | Core power bus. Powers F18A computers, and parts of I/O circuitry (such as registers) that are internal to them. |  |  |  |
|  |  | 17 |  |  |  |  |  |
|  |  | 29 |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |
|  |  | 49 |  |  |  |  |  |
|  |  | 62 |  |  |  |  |  |
|  |  | 75 |  |  |  |  |  |
|  |  | 83 |  |  |  |  |  |
| Power | VDDI | 4 | J11.2 | I/O power bus. Powers I/O pads including the parts of the I/O circuitry collocated with the pads. Includes analog pads for nodes 117 and 617. |  |  |  |
|  |  | 19 |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |
|  |  | 47 |  |  |  |  |  |
|  |  | 64 |  |  |  |  |  |
|  |  | 82 |  |  |  |  |  |
| Power | VDDA | 74 | J11.2 | Analog power bus for pads of nodes 709, 713 and 717. |  |  |  |
| Ground | GND | DAP | any gnd | Common ground and heat sink. |  |  |  |

### Target Chip

| Type | Name | Pin | Access | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bus I/O | d00 | 1 | J30.1 | Bits 0 through 17 of node 007 UP port. General purpose bidirectional parallel bus, such as external memory data. |  |  |  |
|  | d01 | 2 | J30.2 |  |  |  |  |
|  | d02 | 3 | J30.3 |  |  |  |  |
|  | d03 | 8 | J30.6 |  |  |  |  |
|  | d04 | 9 | J30.7 |  |  |  |  |
|  | d05 | 10 | J30.8 |  |  |  |  |
|  | d06 | 11 | J30.9 |  |  |  |  |
|  | d07 | 12 | J30.10 |  |  |  |  |
|  | d08 | 13 | J30.11 |  |  |  |  |
|  | d09 | 16 | J30.12 |  |  |  |  |
|  | d10 | 21 | J30.15 |  |  |  |  |
|  | d11 | 22 | J30.16 |  |  |  |  |
|  | d12 | 23 | J31.1 |  |  |  |  |
|  | d13 | 24 | J31.2 |  |  |  |  |
|  | d14 | 25 | J31.3 |  |  |  |  |
|  | d15 | 30 | J31.4 |  |  |  |  |
|  | d16 | 31 | J31.5 |  |  |  |  |
|  | d17 | 32 | J31.6 |  |  |  |  |
| GPIO | 008.17 | 33 | J31.7 | General purpose 4-pin node. Might be used for memory or bus control and handshake lines. |  |  |  |
|  | 008.5 | 34 | J31.8 |  |  |  |  |
|  | 008.3 | 35 | J31.9 |  |  |  |  |
|  | 008.1 | 36 | J31.10 |  |  |  |  |
| Bus I/O | a17 | 37 | J31.11 | Bits 17 through 0 of node 009 UP port. General purpose bidirectional parallel bus, such as external memory address. |  |  |  |
|  | a16 | 38 | J31.12 |  |  |  |  |
|  | a15 | 39 | J31.13 |  |  |  |  |
|  | a14 | 42 | J31.14 |  |  |  |  |
|  | a13 | 43 | J31.15 |  |  |  |  |
|  | a12 | 44 | J31.16 |  |  |  |  |
|  | a11 | 45 | J36.15 |  |  |  |  |
|  | a10 | 46 | J36.14 |  |  |  |  |
|  | a09 | 53 | J36.9 |  |  |  |  |
|  | a08 | 54 | J36.8 |  |  |  |  |
|  | a07 | 55 | J36.7 |  |  |  |  |
|  | a06 | 56 | J36.6 |  |  |  |  |
|  | a05 | 57 | J36.5 |  |  |  |  |
|  | a04 | 58 | J36.4 |  |  |  |  |
|  | a03 | 65 | J32.15 |  |  |  |  |
|  | a02 | 66 | J32.14 |  |  |  |  |
|  | a01 | 67 | J32.13 |  |  |  |  |
|  | a00 | 68 | J32.12 |  |  |  |  |
| SERDES | 001.17 | 27 | S17 | Node 001 Clock |  |  | Connected to Host node 701 SERDES. Both chips reset to SDERDES boot. |
|  | 001.1 | 26 | S1 | Node 001 Data |  |  |  |
| SERDES | 701.17 | 86 | J28.2 | Node 701 Clock |  |  | Available for experimentation. |
|  | 701.1 | 87 | J28.4 | Node 701 Data |  |  |  |
| GPIO | 300.17 | 14 | J35.2 | Sync clock | General purpose 2-pin node. ROM supports synchronous boot. May be connected to Host node 300. |  |  |
|  | 300.1 | 15 | J34.2 | Sync data |  |  |  |
| GPIO | 708.17 | 78 | J23.10 | Rx Input |  | General purpose 2-pin node. ROM supports asynchronous boot. May be connected to USB port C for IDE operations. |  |
|  | 708.1 | 79 | J23.12 | Tx Out |  |  |  |
| GPIO | 705.17 | 85 | J32.1 | Data In |  | General purpose 4-pin node. If 705.17 is low on reset, ROM will attempt SPI memory boot using signal assignments shown, driving signals on 705.5, 3, 1, and will leave these in output mode unless programmed otherwise. |  |
|  | 705.5 | 84 | J32.2 | Data Out |  |  |  |
|  | 705.3 | 81 | J32.3 | Chip Enable- |  |  |  |
|  | 705.1 | 80 | J32.4 | Clock |  |  |  |
| GPIO | 100.17 | 20 | J30.14 | General purpose 1-pin nodes. No special ROM or interconnections. |  |  |  |
|  | 200.17 | 18 | J30.13 |  |  |  |  |
|  | 500.17 | 7 | J30.5 |  |  |  |  |
|  | 600.17 | 6 | J30.4 |  |  |  |  |
|  | 317.17 | 52 | J36.10 |  |  |  |  |
|  | 417.17 | 59 | J36.3 |  |  |  |  |
| Analog In | 709.ai | 76 | J32.6 | Analog nodes whose I/O is powered by separate VDDA bus. |  |  |  |
| Analog Out | 709.ao | 77 | J32.5 |  |  |  |  |
| Analog In | 713.ai | 73 | J32.7 |  |  |  |  |
| Analog Out | 713.ao | 72 | J32.8 |  |  |  |  |
| Analog In | 717.ai | 69 | J32.11 |  |  |  |  |
| Analog Out | 717.ao | 70 | J32.10 |  |  |  |  |
| GPIO | 715.17 | 71 | J32.9 | General purpose 1-pin node whose pin is shared (read only) by the above analog nodes and may be used by them for timing or other purposes. |  |  |  |
| Analog In | 617.ai | 61 | J36.1 | Analog node whose I/O is powered by VDDI bus. |  |  |  |
| Analog Out | 617.ao | 63 | J32.16 |  |  |  |  |
| GPIO | 517.17 | 60 | J36.2 | General purpose 1-pin node whose pin is shared (read only) by Analog 617. |  |  |  |
| Analog In | 117.ai | 48 | J36.13 | Analog node whose I/O is powered by VDDI bus. |  |  |  |
| Analog Out | 117.ao | 50 | J36.12 |  |  |  |  |
| GPIO | 217.17 | 51 | J36.11 | General purpose 1-pin node whose pin is shared (read only) by Analog 117. |  |  |  |
| Input | RESET- | 88 | J22.2 | Reset signal, active low. Also pins J22.4 and 6. |  |  |  |
| Power | VDDC | 5 | J14.2 | Core power bus. Powers F18A computers, and parts of I/O circuitry (such as registers) that are internal to them. |  |  |  |
|  |  | 17 |  |  |  |  |  |
|  |  | 29 |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |
|  |  | 49 |  |  |  |  |  |
|  |  | 62 |  |  |  |  |  |
|  |  | 75 |  |  |  |  |  |
|  |  | 83 |  |  |  |  |  |
| Power | VDDI | 4 | J15.2 | I/O power bus. Powers I/O pads including the parts of the I/O circuitry collocated with the pads. Includes analog pads for nodes 117 and 617. |  |  |  |
|  |  | 19 |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |
|  |  | 47 |  |  |  |  |  |
|  |  | 64 |  |  |  |  |  |
|  |  | 82 |  |  |  |  |  |
| Power | VDDA | 74 | J16.2 | Analog power bus for pads of nodes 709, 713 and 717. |  |  |  |
| Ground | GND | DAP | any gnd | Common ground and heat sink. |  |  |  |